Draft PDS-01

HERS Addendum 103, Direct Expansion HP & AC Modeling Supplement

# Modify the ANSI/RESNET/ICC 301-2022 Appendix C by adding the following:

# C.x Direct Expansion Modeling

## C.x.10 Defrost

The integrated heating capacity and power of the heat pump shall be reduced for any outdoor drybulb temperature lower than the maximum defrost temperature (40°F) using the equations below.

fdef(Todb) = max(min(0.134-0.003*Todb, 0.08), 0)

Qgross,int,htg,i = Qgross,ss,htg,i*[1 - 1.8*fdef(Todb)]

Pgross,int,htg,i = Pgross,ss,htg,i*[1 - 0.3*fdef(Todb)]

While defrosting in systems with supplemental heat, the supplemental heat is active and the indoor fan is running during the defrost period. The supplemental heater is cycled or modulated to add exactly enough heat to offset the reduced capacity.

Pan heaters are assumed to operate at 150W whenever the outdoor temperature is below 32°F and above the minimum compressor operating temperature of the equipment.

## C.x.14 Minimum Rated Features

| Feature | Description | Source | Default | Reference Home Value |
| --- | --- | --- | --- | --- |
| Qnet,tot,clg,A,full | Rated full load net total cooling capacity at AHRI “A” (95°F) conditions | AHRI Certificate/Directory | Required | For Air Conditioners: Sized to meet the sensible cooling load. For Heat Pump equipment: Sized to equal the larger of the building heating and sensible cooling loads. |
| SEER2 | Seasonal Energy Efficiency Ratio as defined by AHRI 210/240-2023 | AHRI Certificate | Required | 12.35 |
| EER2 | Energy Efficiency Ratio at AHRI “A” (95°F) conditions as defined by AHRI 210/240-2023 | AHRI Certificate | Required | 10.7 |
| Qnet,ss,htg,H1,full | Rated full load net steady-state heating capacity at AHRI “H1” (47°F) conditions | AHRI Certificate | Required | Sized to equal the larger of the building heating and sensible cooling loads. For Electric Resistance Supplemental Heating: The building heating sensible load shall be calculated at the greater of the minimum compressor operating temperature and the heating design temperature for the location. For Fossil Fuel Supplemental Heating: The building heating sensible load shall be calculated at the greater of the heating design temperature for the location and 25°F. |
| Qnet,ss,htg,H3,full | Rated full load net steady-state heating capacity at AHRI “H3” (17°F) conditions | AHRI Certificate | Required | 0.626* Qnet,ss,htg,H1,full |
| HSPF2 | Heating Seasonal Performance Factor as defined by AHRI 210/240-2023 | AHRI Certificate | Required | 6.55 |
| Number of Speeds/stages | Number of speeds/stages used in compressor control: Single Stage Two Stage Variable Capacity | Product Literature | Required | Single Stage |
| Fan Motor Type | Type of motor used by the indoor fan | Product Literature | Permanent Split Capacitor (PSC) for single speed equipment, otherwise Brushless Permanent Magnet (BPM) | PSC |
| Type of Supplemental Heating | Electric Resistance Fossil Fuel Furnace None | Product literature, visual inspection | Electric Resistance | Same as Rated Home |
| Minimum Compressor Operating Temperature | Minimum outdoor temperature for compressor heating operation | For Electric Resistance Supplemental Heating: Product Literature, NEEP Database Lowest Catalogued Temperature (LCT) For Fossil Fuel Supplemental Heating: Fixed at 430°F | For Electric Resistance Supplemental Heating: Number of Speeds <= 2: 05°F Number of Speeds > 2: -2010°F For Fossil Fuel Supplemental Heating: No Default | For Electric Resistance Supplemental Heating: 05°F For Fossil Fuel Supplemental Heating: 40°F |

(Informative) Based on Central Valley Research Homes project data