Comment/Explanation*: 
Include your justification for your proposed change to the draft standard below.
_________________________________________________________________________________________

I disagree with the informative note describing brush-based door sweeps as acceptable.  I argue that brush-based sweeps do not provide significant resistance to air flow.  The blower door frame will make a better seal with the floor than a brush-based sweep, leaving the air leakage understated.

Accordingly, I have struck the part of the note referencing brush-based door sweeps.

Proposed Change to the Draft Standard*
Use “strikethrough” and “underline” formatting to indicate all proposed changes. Changes must be shown with “hard-formatting” strikethrough and underline, not “track changes”.

Use a color other than red to indicate proposed changes to the draft.
____________________________________________________________________________________________

403.2.4 The door where the blower door is installed shall be inspected for the presence of a door seal on all four edges of a door, which is properly installed per manufacturer’s instructions to minimize air leakage between the door and door frame. Where such seal is not present or is not properly installed, 140 CFM50 shall be added to the measured airflow. This adjustment as well as the presence, installation quality and condition of the door seal shall be documented in the final test report.

(Informative Note) The door seal at the bottom can include door shoes as well as brush-based door sweeps.

(Normative Note) The adjustment may be subsequently removed if the door seal continuity is inspected and confirmed.